#### Seat Cushion, Rear

Sedan

Removal

- Apply knee pressure to lower front portion of the rear seat cushion. Push rearward to disengage rear seat cushion pad and frame (600A88) from rear seat cushion front retainers (63370).

- Remove rear seat cushion from vehicle.

Installation

- Position rear seat cushion assembly into vehicle.

- Place rear safety belt and buckles (60045) on top of rear seat cushion.

- Apply knee pressure to lower portion of rear seat cushion pad and frame. Push rearward and down to lock the rear seat cushion pad and frame into position.

- Pull rear seat cushion pad and frame forward to make sure it is secured into rear seat cushion front retainer.

Cushion, Rear Seat Sedan

#### Seat Back, Rear

60/40 Split Bench—Sedan

Removal

- NOTE: Safety belt bolts do not secure seat back to vehicle.

Remove rear seat cushion as described.

- Fold down back seat and remove two bolts in the center of each seat back striker plates.

- Remove two bolts from outboard rear seat back frame retaining brackets.

- Lift seat back up and pull forward to remove from vehicle.

Installation

- Follow removal procedure in reverse order.

- Tighten bolts to 19-26 Nm (14-19 lb-ft).

Rear Seat Back—Sedan

#### Trim Panel, Package Tray

Removal

- Remove rear seat as described in Section 01-10 .

- Remove upper quarter trim panels as described.

- Remove package tray trim panel (46668) by pulling package tray trim panel forward and upward.

Installation

- Follow removal procedure in reverse order.